Technion

Electrical Engineering Department

High Speed Digital System Lab

Memory Management

Documentation

Version: 1.0

Last update: 28.04.2011

Written By: Beeri Schreiber and Alon Yavich

# Table of content

Table of Changes	3

Scope	3

Abbreviations	3

General Description	3

Memory Management Pinout	4

Memory Management Generic Parameters	5

Mem_Ctrl_Wr Pinout	6

Mem_Ctrl_Wr Generic Parameters	7

Mem_Ctrl_Rd Pinout	7

Mem_Ctrl_Rd Generic Parameters	8

Arbiter Pinout	9

Arbiter Generic Parameters	10

Resources	10

Memory Management Waves	11

Mem_Ctrl_Wr Waves	11

Burst	11

Summary Chunk	12

Debug Mode	13

Cycle Split	14

Mem_Ctrl_Wr Waves	15

Burst	15

Restart from initial address	16

Cycle Split	17

Arbiter Waves	18

Performed Tests for Memory Management	19

Performed Test for Mem_Ctrl_Wr	19

Performed Test for Mem_Ctrl_Rd	19

# Table of Tables

Table 1 – Table of Changes	3

Table 2 – Memory Management Pinout	5

Table 3 – Memory Management Parameters	5

Table 4 – mem_ctrl_wr Pinout	6

Table 5 – mem_ctrl_wr Parameters	7

Table 6 – mem_ctrl_rd Pinout	8

Table 7 – Memory Management Parameters	8

Table 8 – Arbiter Pinout	10

Table 9 – Arbiter Generic Parameters	10

# Tables of Figures

Figure 1 - General Scheme	3

Figure 2 – Memory Management Pinout	4

Figure 3 – mem_ctrl_wr Pinout	6

Figure 4 – mem_ctrl_rd Pinout	7

Figure 5 – Arbiter Pinout	9

Figure 6 – Mem_Ctrl_Wr Burst	11

Figure 7 – Mem_Ctrl_Wr Summary Chunk	12

Figure 8 – Mem_Ctrl_Wr Debug Mode	13

Figure 9 – Mem_Ctrl_Wr Cycle Split	14

Figure 10 – Mem_Ctrl_Rd Burst	15

Figure 11 – Mem_Ctrl_Rd Burst	16

Figure 12 – Mem_Ctrl_Rd Cycle Split	17

Figure 13 – Arbiter	18

# Table of Changes

| Version | Date of Change | Description |
| --- | --- | --- |
| 1.0 | 28.04.2011 | Creation of documentation |

Table 1 – Table of Changes

# Scope

This document aims to describe the working method of the Memory Management Block.

# Abbreviations

- SDRAM – Synchronous Dynamic Random Access Memory

- RAS – Row Address

- CAS – Column Address

- WBM – Wishbone Master

- WBS – Wishbone Slave

- TB – Test bench

# General Description

Figure 1 - General Scheme

The Memory Management block manages the write and read transaction to and from the SDRAM, through the SDRAM Controller.

The Memory Management block is consumed of the following components:

- Mem_Ctrl_Wr – Responsible to receive data, through WBS, and store it, using SDRAM controller, to the SDRAM.
Data (up to 1KByte) is received from WBS (1) and stored into internal RAM (R1). As soon as half of the burst has been transmitted (Input RAM is 8 bits, while input SDRAM is 16 bits, therefore RAM transaction and SDRAM transaction will end at about the same time), SDRAM transaction is initialized, which stores the data from the internal RAM to the SDRAM, through the SDRAM Controller.

- Mem_Ctrl_Rd – Responsible to transmit data, using SDRAM controller, to the data requester, through WBS.
As soon as some words (about 10) are written to the internal RAM (R2) from the SDRAM, the WBS (2) negates STALL_O and transmits data from the internal RAM.

- Arbiter – Toggles between Mem_Ctrl_Wr WBM (B) and Mem_Ctrl_Rd WBM (C), where the Mem_Ctrl_Wr has higher priority, to prevent data loss.

- Wr_rd_bank – Holds the MSB address bit of the SDRAM write / read address, which is actually the double bank. While mem_ctrl_wr writes to bank '10', mem_ctrl_rd reads from bank '00'.

- Registers – Hold necessary data for above blocks:

  - Type Register – Type of message (Address 1h).

  - Debug Address Register – Write / Read SDRAM address for SDRAM in Debug mode (Address 2h4h).

# Memory Management Pinout

Figure 2 – Memory Management Pinout

| Pin Name | Direction | Description |
| --- | --- | --- |
| Clk_i | In | Wishbone Clock (133MHz) |
| Rst | In | Reset |
| Wr_wbs_adr_i[9..0] | In | WBS Address (to mem_ctrl_wr) |
| Wr_wbs_tga_i[9..0] | In | WBS Tag Address – Burst length (to mem_ctrl_wr) |
| Wr_wbs_dat_i[7..0] | In | WBS Input Data (to mem_ctrl_wr) |
| Wr_wbs_cyc_i | In | WBS Cycle (to mem_ctrl_wr) |
| Wr_wbs_stb_i | In | WBS Strobe (to mem_ctrl_wr) |
| Wr_wbs_we_i | In | WBS Write Enable (to mem_ctrl_wr) |
| Wr_wbs_tgc_i | In | WBS Tag Cycle – Write to components ('0') or registers ('1') (to mem_ctrl_wr) |
| Rd_wbs_adr_i[9..0] | In | WBS Address (to mem_ctrl_rd) |
| Rd_wbs_tga_i[9..0] | In | WBS Tag Address – Burst length (to mem_ctrl_rd) |
| Rd_wbs_cyc_i | In | WBS Cycle (to mem_ctrl_rd |
| Rd_wbs_stb_i | In | WBS Strobe (to mem_ctrl_rd) |
| Rd_wbs_tgc_i | In | WBS Tag Cycle – Write to components ('0') or registers ('1') (to mem_ctrl_rd) |
| Wbm_dat_i[15..0] | In | WBM Input Data (From SDRAM Controller) |
| Wbm_stall_i | In | WBM Stall – Repeat last strobe (From SDRAM Controller) |
| Wbm_err_i | In | WBM Error (From SDRAM Controller) |
| Wbm_ack_i | In | WBM Acknowledged (From SDRAM Controller) |
| Wr_wbs_dat_o[7..0] | Out | WBS Output Data (Registers' value) |
| Wr_wbs_stall_o | Out | WBS Stall – Repeat last strobe (Payload / Registers) |
| Wr_wbs_ack_o | Out | WBS Acknowledged (Payload / Registers) |
| Wr_wbs_err_o | Out | WBS Error (From mem_ctrl_wr) |
| Rd_wbs_dat_o[7..0] | Out | WBS Output Data, from SDRAM (From mem_ctrl_rd) |
| Rd_wbs_stall_o | Out | WBS Stall (From mem_ctrl_rd) |
| Rd_wbs_ack_o | Out | WBS Acknowledged (From mem_ctrl_rd) |
| Rd_wbs_err_o | Out | WBS Error (From mem_ctrl_rd) |
| Wbm_adr_o[21..0] | Out | WBM Address to SDRAM Controller |
| Wbm_dat_o[15..0] | Out | WBM Data to SDRAM Controller |
| Wbm_we_o | Out | WBM Write Enable to SDRAM Controller |
| Wbm_tga_o[7..0] | Out | WBM Tag Address – Burst length to SDRAM Controller |
| wbm_cyc_o | Out | WBM Cycle to SDRAM Controller |
| Wbm_stb_o | Out | WBM Strobe to SDRAM Controller |

Table 2 – Memory Management Pinout

# Memory Management Generic Parameters

| Generic Parameter | Default Value | Description |
| --- | --- | --- |
| Reset_polartiy_g | '0' | Reset active in this polarity |
| mode_g | 0 | Relevant bit in type register, which represent Normal ('0') or Debug ('1') mode |
| message_g | 1 | Relevant bit in type register, which represent Image chunk ('0') or Summary chunk ('1') mode |
| img_hor_pixels_g | 640 | Horizontal Pixels in image |
| img_ver_lines_g | 480 | Vertical lines in image |

Table 3 – Memory Management Parameters

# Mem_Ctrl_Wr Pinout

Figure 3 – mem_ctrl_wr Pinout

| Pin Name | Direction | Description |
| --- | --- | --- |
| Clk_i | In | Wishbone Clock (133MHz) |
| Rst | In | Reset |
| wbs_adr_i[9..0] | In | WBS Address |
| wbs_tga_i[9..0] | In | WBS Tag Address – Burst length |
| wbs_dat_i[7..0] | In | WBS Input Data |
| wbs_cyc_i | In | WBS Cycle |
| wbs_stb_i | In | WBS Strobe |
| Wbm_stall_i | In | WBM Stall – Repeat last strobe (From SDRAM Controller) |
| Wbm_err_i | In | WBM Error (From SDRAM Controller) |
| Wbm_ack_i | In | WBM Acknowledged (From SDRAM Controller) |
| Arbiter_gnt | In | Grant on SDRAM Controller from Arbiter |
| Bank_val | In | SDRAM's bank (1 / 0) |
| Type_reg[7..0] | In | Type Register value |
| Wr_addr_reg | In | Debug address register value |
| wbs_stall_o | Out | WBS Stall – Repeat last strobe |
| wbs_ack_o | Out | WBS Acknowledged |
| wbs_err_o | Out | WBS Error |
| Wbm_adr_o[21..0] | Out | WBM Address to SDRAM Controller |
| Wbm_dat_o[15..0] | Out | WBM Data to SDRAM Controller |
| Wbm_we_o | Out | WBM Write Enable to SDRAM Controller |
| Wbm_tga_o[7..0] | Out | WBM Tag Address – Burst length to SDRAM Controller |
| wbm_cyc_o | Out | WBM Cycle to SDRAM Controller |
| Wbm_stb_o | Out | WBM Strobe to SDRAM Controller |
| Arbiter_req | Out | Request for grant on SDRAM Controller from Arbiter |
| Bank_switch | Out | '1' to switch between SDRAM banks (bank_val) |
| Wr_cnt_val[18..0] | Out | Number of words, written so far to SDRAM |
| Wr_cnt_en | Out | All image has been transmitted to SDRAM, and wr_cnt_val value is valid |

Table 4 – mem_ctrl_wr Pinout

# Mem_Ctrl_Wr Generic Parameters

| Generic Parameter | Default Value | Description |
| --- | --- | --- |
| Reset_polartiy_g | '0' | Reset active in this polarity |
| mode_g | 0 | Relevant bit in type register, which represent Normal ('0') or Debug ('1') mode |
| message_g | 1 | Relevant bit in type register, which represent Image chunk ('0') or Summary chunk ('1') mode |
| img_hor_pixels_g | 640 | Horizontal Pixels in image |
| img_ver_lines_g | 480 | Vertical lines in image |

Table 5 – mem_ctrl_wr Parameters

# Mem_Ctrl_Rd Pinout

Figure 4 – mem_ctrl_rd Pinout

| Pin Name | Direction | Description |
| --- | --- | --- |
| Clk_i | In | Wishbone Clock (133MHz) |
| Rst | In | Reset |
| wbs_adr_i[9..0] | In | WBS Address |
| wbs_tga_i[9..0] | In | WBS Tag Address – Burst length |
| wbs_cyc_i | In | WBS Cycle |
| wbs_tgc_i | In | WBS Tag Cycle. '1' to start reading from the first address in the chosen bank (according to bank_val) |
| wbs_stb_i | In | WBS Strobe |
| Wbm_dat_i[15..0] | In | WBM Data from SDRAM Controller |
| Wbm_stall_i | In | WBM Stall – Repeat last strobe (From SDRAM Controller) |
| Wbm_err_i | In | WBM Error (From SDRAM Controller) |
| Wbm_ack_i | In | WBM Acknowledged (From SDRAM Controller) |
| Arbiter_gnt | In | Grant on SDRAM Controller from Arbiter |
| Bank_val | In | SDRAM's bank (1 / 0) |
| Type_reg[7..0] | In | Type Register value |
| rd_addr_reg | In | Debug address register value |
| rd_cnt_val[18..0] | In | Number of words, written to SDRAM |
| rd_cnt_en | In | All image has been transmitted to SDRAM, and wr_cnt_val value is valid |
| wbs_dat_o[7..0] | In | WBS Output Data |
| wbs_stall_o | Out | WBS Stall – Repeat last strobe |
| wbs_ack_o | Out | WBS Acknowledged |
| wbs_err_o | Out | WBS Error |
| Wbm_adr_o[21..0] | Out | WBM Address to SDRAM Controller |
| Wbm_we_o | Out | WBM Write Enable to SDRAM Controller |
| Wbm_tga_o[7..0] | Out | WBM Tag Address – Burst length to SDRAM Controller |
| wbm_cyc_o | Out | WBM Cycle to SDRAM Controller |
| Wbm_stb_o | Out | WBM Strobe to SDRAM Controller |
| Arbiter_req | Out | Request for grant on SDRAM Controller from Arbiter |

Table 6 – mem_ctrl_rd Pinout

# Mem_Ctrl_Rd Generic Parameters

| Generic Parameter | Default Value | Description |
| --- | --- | --- |
| Reset_polartiy_g | '0' | Reset active in this polarity |
| mode_g | 0 | Relevant bit in type register, which represent Normal ('0') or Debug ('1') mode |
| img_hor_pixels_g | 640 | Horizontal Pixels in image |
| img_ver_lines_g | 480 | Vertical lines in image |

Table 7 – Memory Management Parameters

# Arbiter Pinout

Figure 5 – Arbiter Pinout

| Pin Name | Direction | Description |
| --- | --- | --- |
| Clk | In | Clock (133MHz) |
| Rst | In | Reset |
| Wr_req | In | Request for grant on SDRAM controller from mem_ctrl_wr |
| Rd_req | In | Request for grant on SDRAM controller from mem_ctrl_rd |
| Wr_wbm_adr_o[21..0] | In | WBM Address to SDRAM Controller (From mem_ctrl_wr) |
| Wr_wbm_dat_o[15..0] | In | WBM Data to SDRAM Controller (From mem_ctrl_wr) |
| Wr_wbm_we_o | In | WBM Write Enable to SDRAM Controller (From mem_ctrl_wr) |
| Wr_wbm_tga_o[7..0] | In | WBM Tag Address – Burst length to SDRAM Controller (From mem_ctrl_wr) |
| Wr_wbm_cyc_o | In | WBM Cycle to SDRAM Controller (From mem_ctrl_wr) |
| Wr_wbm_stb_o | In | WBM Strobe to SDRAM Controller (From mem_ctrl_wr) |
| Rd_wbm_adr_o[21..0] | In | WBM Address to SDRAM Controller (From mem_ctrl_rd) |
| Rd_wbm_dat_o[15..0] | In | WBM Data to SDRAM Controller (From mem_ctrl_rd) |
| Rd_wbm_we_o | In | WBM Write Enable to SDRAM Controller (From mem_ctrl_rd) |
| Rd_wbm_tga_o[7..0] | In | WBM Tag Address – Burst length to SDRAM Controller (From mem_ctrl_rd) |
| Rd_wbm_cyc_o | In | WBM Cycle to SDRAM Controller (From mem_ctrl_rd) |
| Rd_wbm_stb_o | In | WBM Strobe to SDRAM Controller (From mem_ctrl_rd) |
| Wbm_dat_i[15..0] | In | WBM Input data from SDRAM Controller |
| Wbm_stall_i | In | WBM Stall from SDRAM Controller – Repeat last transaction |
| Wr_gnt | Out | Grant on SDRAM Controller to mem_ctrl_wr |
| Rd_gnt | Out | Grant on SDRAM Controller to mem_ctrl_rd |
| Wr_wbm_stall_i | Out | WBM Stall from SDRAM Controller to mem_ctrl_wr |
| Wr_wbm_err_i | Out | WBM error from SDRAM Controller to mem_ctrl_wr |
| Wr_wbm_ack_i | Out | WBM acknowledged from SDRAM Controller to mem_ctrl_wr |
| Rd_wbm_dat_i[15..0] | Out | Output data from SDRAM Controller to mem_ctrl_rd |
| Rd_wbm_stall_i | Out | WBM Stall from SDRAM Controller to mem_ctrl_rd |
| Rd_wbm_err_i | Out | WBM error from SDRAM Controller to mem_ctrl_rd |
| Rd_wbm_ack_i | Out | WBM acknowledged from SDRAM Controller to mem_ctrl_rd |
| Wbm_adr_o[21..0] | Out | WBM Write address to SDRAM Controller |
| Wbm_we_o | Out | WBM Write Enable to SDRAM Controller |
| Wbm_tga_o[7..0] | Out | WBM Tag Address – Burst length to SDRAM Controller |
| wbm_cyc_o | Out | WBM Cycle to SDRAM Controller |
| Wbm_stb_o | Out | WBM Strobe to SDRAM Controller |
| Wbm_dat_o[15..0] | Out | WBM Output data to SDRAM Controller |

Table 8 – Arbiter Pinout

# Arbiter Generic Parameters

| Generic Parameter | Default Value | Description |
| --- | --- | --- |
| Reset_polartiy_g | '0' | Reset active in this polarity |

Table 9 – Arbiter Generic Parameters

# Resources

Required resources, when synthesizing, using Synplify, for Altera's Cyclon II FPGA:

- 1162 Combinational Logic

- 32 EC (Embedded Cell: M4K, which is the RAM)

- 549 FF

Maximum frequency:

148.4 MHz

# Memory Management Waves

## Mem_Ctrl_Wr Waves

### Burst

- 1030 bytes burst to SDRAM is split, by Data Provider, to 1024 bytes burst, and then 6 bytes burst.

- Internal logic writes the data to the SDRAM in 256 words bursts.

- Summary chunk, with total transmitted bytes (1030) is compared with total stored words * 2 to SDRAM (1030 = 515 * 2)

- SDRAM Banks are switched ('0' to '1')

Figure 6 – Mem_Ctrl_Wr Burst

### Summary Chunk

- Type register, bit number 1, is changed to '1', to indicate that summary chunk is being transmitted.

- Received data is calculated, and compared with 2 * total sent words to SDRAM.

- The two values are equal  Bank switch is executed.

Figure 7 – Mem_Ctrl_Wr Summary Chunk

### Debug Mode

- Type register, bit number 0 is set to '1', to indicate on Debug Mode.

- Write Address register is set to the required start address, to write to the SDRAM from.

- WBS received data to internal RAM.

- Transaction is being initialized, to store the data from the internal RAM to the SDRAM.

Figure 8 – Mem_Ctrl_Wr Debug Mode

### Cycle Split

- End of SDRAM Page has been detected (End of burst to SDRAM by SDRAM controller).

- WBM closes the cycle, to split burst, and re-initialize it from where it stops.

- New SDRAM burst is being executed, from the new page.

Figure 9 – Mem_Ctrl_Wr Cycle Split

## Mem_Ctrl_Wr Waves

### Burst

- Data Requester initializes Wishbone Read Cycle

- End of Write Cycle of mem_ctrl_rd causes to banks switch

- Request for grant from Arbiter on SDRAM, and Arbiter grants

- Data from SDRAM is read, and stored into internal 1KB RAM

- Since the burst is 1KB, and SDRAM page is 512Byte, the SDRAM burst is split to two burst

- All data has been acknowledged by data requester, which closes the burst

Figure 10 – Mem_Ctrl_Rd Burst

### Restart from initial address

When an image should be presented on the screen, the data should be read from the beginning of the SDRAM bank, and not from the current point. Therefore, the Data Requester can assert WBM_TGC_O, no indicate that SDRAM should provide data from the first bank's address.

- Cycle Tag is received, therefore current address in SDRAM's read bank is changed to 0h.

- 1KB of data is read.

Figure 11 – Mem_Ctrl_Rd Burst

### Cycle Split

Since SDRAM page is 256 words (=512Bytes), 1KB burst must be split to two burst.

- Precharge, to end read transaction

- End and Start of SDRAM transaction

- RAS, CAS and WE, to start new SDRAM transaction

Figure 12 – Mem_Ctrl_Rd Cycle Split

## Arbiter Waves

- When no instantiation request for grant on SDRAM, Mem_Ctrl_Wr grants the control on the SDRAM.

- Mem_Ctrl_Rd grants the control on the SDRAM, when it request for the grant, and as soon as the request from Mem_Ctrl_Wr is being negated.

Figure 13 – Arbiter

# Performed Tests for Memory Management

## Performed Test for Mem_Ctrl_Wr

- 2 bytes burst

- 4 bytes burst

- 6 bytes burst

- 1024 bytes burst

- 1026 bytes burst

- 1030 bytes burst

- Two 1030 bytes bursts

- Two burst, with 1 clock cycle separates between them.

- 500 bytes burst, in Debug mode, from address 2000decimal = 7D0hex

- Summary chunk after 1030 burst, to switch banks

- Summary chunk after two 1030 bursts, to switch banks

- Error injections:

  - Incorrect summary chunk after 1030 burst. Bank has not been switched

  - Error from SDRAM controller (WBM_ERR_I) caused end of cycle

  - Incorrect address to WBS (expecting sequential addressing, starting from 0) causes WBS_ERR_O signal to rise

## Performed Test for Mem_Ctrl_Rd

- 2 bytes burst

- 4 bytes burst

- 6 bytes burst

- 1024 bytes burst

- 1026 bytes burst

- 1030 bytes burst

- Two 1030 bytes bursts

- Two burst, with 1 clock cycle separates between them.

- 500 bytes burst, in Debug mode, from address 2000decimal = 7D0hex

- Error injections:

  - Error from SDRAM controller (WBM_ERR_I) caused end of cycle

  - Incorrect address to WBS (expecting sequential addressing, starting from 0) – Waits for the next correct address.